AI-Assisted Architectural Study of a SKY130 130 nm Phase-Locked Loop: Block Analysis, Toolchain Setup, and Debugging Strategy

Student Author  |  Week 1 Research Report

Department of Electronics Engineering  ·  Open-Source VLSI Design Programme

Abstract—This report documents a structured, AI-assisted study of a 130 nm SKY130-process phase-locked loop (PLL) implemented in the open-source reference repository nitjsr_pll_130nm. The objective of Week 1 is conceptual mastery rather than full reproduction: understanding PLL sub-block architecture, feedback loop mathematics, toolchain requirements (xschem / ngspice / Magic / OpenLane), pre-layout versus post-layout simulation differences, and systematic debugging strategies. Every insight recorded here has been independently verified against the reference repository and primary literature.

I. Introduction

Phase-Locked Loops are essential mixed-signal circuits used for clock synthesis, frequency multiplication, and jitter reduction. The reference repository targets an 8× clock multiplier on the SKY130 open-source PDK: a 5 MHz reference input is multiplied to produce a 40 MHz locked output [1]. The entire flow uses only free and open-source tools, making it an ideal learning platform.

This week's work decomposes the repository into independently understandable tasks, uses AI prompting to accelerate reading of netlist and simulation files, and produces a verified block-by-block understanding of what inputs, outputs, and failure modes to expect.

II. PLL Architecture Overview

The 8× PLL comprises five major sub-blocks connected in a negative-feedback loop. The reference clock f_ref = 5 MHz and the divided-down VCO output f_div are compared by the Phase Frequency Detector (PFD). The resulting UP/DOWN pulses drive the Charge Pump (CP), which sources or sinks current into the Loop Filter (LF). The resulting control voltage V_ctrl tunes the Voltage-Controlled Oscillator (VCO), and a ÷8 Frequency Divider closes the feedback path [1].

Flow: f_ref → PFD → CP → LF → VCO → f_out, with ÷8 Divider feeding back to PFD.

The fundamental frequency relationship is:

f_out = N · f_ref  →  40 MHz = 8 × 5 MHz

III. Sub-Block Analysis

A. Phase Frequency Detector

The PFD is a digital state machine, typically built from two D-flip-flops with an AND-gate reset path. When f_ref leads f_div, it asserts UP; when f_div leads, it asserts DOWN. A key failure mode is the dead zone: if reset propagation is too fast, narrow pulses near lock are suppressed, causing phase noise. In xschem schematics, the PFD is often drawn as a custom cell referencing SKY130 DFF primitives.

B. Charge Pump

The CP converts digital UP/DOWN pulses into an analog current. An ideal CP sources I_CP on UP and sinks the same on DOWN. Mismatch between source and sink currents is a primary cause of reference spurs. In the SKY130 netlist, PMOS transistors act as current sources and NMOS as current sinks, biased by cascode mirrors to improve output impedance.

C. Loop Filter

A passive second-order RC filter (R₁, C₁, C₂) converts CP current pulses to V_ctrl. The zero introduced by R₁C₁ stabilises the loop. The loop bandwidth is approximately:

ω_n = √(K_VCO · I_CP / (2π · N · C₁))

Stability requires phase margin > 45°. Choosing C₂ ≈ C₁/10 keeps the second pole well above ω_n.

D. Voltage-Controlled Oscillator

The VCO is a current-starved ring oscillator with an odd number of inverter stages. V_ctrl sets the tail-current bias, linearly shifting oscillation frequency. The VCO gain K_VCO (MHz/V) is extracted from a V_ctrl DC sweep. Ring oscillators on SKY130 exhibit significant supply-noise sensitivity; a replica-bias stage reduces this.

E. Divide-by-8 Frequency Divider

Three cascaded toggle flip-flops implement ÷2, ÷4, ÷8 division. The output of the third stage feeds f_div back to the PFD. Layout placement of the divider close to the PFD minimises routing skew, which is critical for lock accuracy.

IV. SKY130 PDK & Toolchain Setup

| Tool | Role | Key File / Command |
| --- | --- | --- |
| xschem | Schematic capture | .sch → netlist export |
| ngspice | SPICE simulation | ngspice top.spice |
| Magic VLSI | Layout & DRC | magic -T sky130A.tech |
| netgen | LVS check | netgen -batch lvs |
| OpenLane | RTL→GDS flow | flow.tcl |

PDK setup requires setting the environment variable PDK_ROOT to the SKY130 installation path. The most common setup failure is a version mismatch between the PDK libraries and the installed tools — resolved by pinning versions via the volare PDK manager [2].

V. Pre-Layout vs. Post-Layout Simulation

Pre-layout simulations use ideal wire connections from the xschem netlist and confirm circuit functionality. Post-layout simulations use an RC-extracted netlist generated by Magic's ext2spice tool. Parasitic capacitances on VCO nodes directly degrade oscillation frequency and worsen phase noise. Expected differences: (a) VCO free-running frequency shifts by 5–20%; (b) lock time increases; (c) charge pump glitches may appear due to gate capacitance on switching nodes.

VI. Simulation Workflow & Debug Strategy

Recommended incremental simulation strategy: (1) Verify VCO free-running range with a V_ctrl DC sweep before any loop closure. (2) Test PFD UP/DOWN pulse widths with a two-clock transient stimulus. (3) Verify CP current balance by probing I(V_ctrl) node directly. (4) Close the loop and run a full transient to observe lock. (5) For post-layout, compare V_ctrl settling waveform with pre-layout to quantify parasitic impact.

Common failure modes and fixes:

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| VCO not oscillating | Bias current too low | Increase Vctrl initial condition |
| PLL never locks | fref outside VCO range | Re-tune loop filter / KVCO |
| SPICE convergence error | Parasitic short in netlist | Check ext2spice warnings |
| LVS mismatch | Floating port in layout | Verify all labels in Magic |

VII. Conclusion

This structured decomposition of the nitjsr_pll_130nm repository provides a clear pathway from PLL theory to open-source implementation. Each sub-block has defined inputs, outputs, and testable failure modes. The AI-assisted workflow accelerated literature cross-referencing and netlist reading, while human verification ensured correctness. Weeks 2–4 will proceed with incremental simulation, layout, and full post-extraction verification.

References

[1] H. Singh, "nitjsr_pll_130nm: SKY130 8× Clock Multiplier PLL," GitHub, 2023. [Online]. Available: https://github.com/himansh107/nitjsr_pll_130nm

[2] efabless, "volare: SKY130 PDK version manager," GitHub, 2022. [Online]. Available: https://github.com/efabless/volare

[3] R. Best, Phase-Locked Loops: Design, Simulation, and Applications, 6th ed. McGraw-Hill, 2007.

[4] B. Razavi, Design of Analog CMOS Integrated Circuits, 2nd ed. McGraw-Hill, 2017.